# Comparator Lab Exercise: 
Voltage Monitor

# Overview

In this project you will implement a software based comparator to monitor the voltage from a potentiometer-based divider. You will evaluate the polling software.

# Details

## Hardware

Figure 1. Nucleo-F401RE pinout.

### Connections

Figure 2. Potentiometer circuit forming voltage divider for comparator monitoring.

Build the circuit shown in Figure 2 on your breadboard.  Connect the potentiometer signals to MCU board as shown in table below. This matches the pins used in the lab code.

Table 1. Signals and connections

| Signal Name | Description | Direction | MCU |  |
| --- | --- | --- | --- | --- |
| P3V3 | Positive supply voltage | Power |  |  |
| GND | Ground | Power |  |  |
| VCompNeg | Variable input voltage | Input to MCU | PA_0 |  |
| VCompPlus | Fixed voltage reference | Input to MCU | PA_1 |  |

- Compile the supplied lab software for polling operation, load it onto the MCU and run it.

- Measure the 3V3 supply rail, which is divided for the comparator’s negative reference voltage. 
V3v3. = ___________V.

- At what input voltage (from the potentiometer) does the LED change color? Use a multimeter to measure this voltage.

- At what input voltage do you expect the LED to change color, given theinput to VCompNeg? Does this match the actual code?

- Change the source code to use interrupts (controlled by the #if in the main function). Verify the code still has the same threshold voltage. What has changed?

- Modify the resistors to change LED color at 1V. Verify that your changes work.